Sorting of numbers in a sequence

Exp 3 (a) : Ascending Order

Aim :

To write and execute an 8085 program to sort a set of numbers in ascending order.

### Apparatus Required:

- 8085 Online Simulator (8085simulator.github.io or similar)

- Test input data

- Instruction set reference

### Algorithm (Ascending Order):

- Load count from memory into register C.

- Subtract 1 from count and store in B (outer loop counter).

- Outer loop (B times):

  - Point HL to the first data.

  - Copy C to D (inner loop counter).

  - Inner loop (D times):

    - Compare adjacent elements.

    - If current > next, swap them.

  - Decrement B and repeat.

- End.

### Assembly Code (Ascending Order):

LXI H, 4200     ; HL points to count

MOV C, M        ; Load count to C

DCR C           ; C = C - 1

MOV B, C        ; B = Outer loop counter

OUTER: LXI H, 4201    ; Point HL to first number

MOV D, C              ; D = Inner loop counter

INNER: MOV A, M       ; A = M[i]

INX H

CMP M                ; Compare A with M[i+1]

JC SKIP              ; If A < M[i+1], skip swap

MOV E, M             ; Swap logic

MOV M, A

DCX H

MOV M, E

INX H

SKIP: DCR D

JNZ INNER

DCR B

JNZ OUTER

HLT

### Sample Input (Memory Starting at 4200H):

| Address | Data |
| --- | --- |
| 4200H | 05H ; Count = 5 |
| 4201H | 3AH |
| 4202H | 12H |
| 4203H | 41H |
| 4204H | 1CH |
| 4205H | 2FH |

### Expected Output (Ascending):

| Address | Data |
| --- | --- |
| 4201H | 12H |
| 4202H | 1CH |
| 4203H | 2FH |
| 4204H | 3AH |
| 4205H | 41H |

Result:  The 8085 assembly language program was successfully executed to sort a given set of numbers in ascending order.

Exp 3 (b) : Descending Order

### Aim:

To write and execute an 8085 program to sort a set of numbers in descending order.

### Apparatus / Tools:

- 8085 Online Simulator

- Hex input data

- Instruction set reference

### Algorithm (Descending Order):

- Load the count of numbers from memory into register C.

- Subtract 1 from count and store in B.

- Outer loop (B times):

  - Point HL to first number.

  - Copy count to D.

  - Inner loop (D times):

    - Compare adjacent numbers.

    - If current < next, swap them.

  - Repeat until sorted.

### Assembly Code (Descending Order):

LXI H, 4200     ; HL points to count

MOV C, M        ; Load count to C

DCR C           ; C = C - 1

MOV B, C        ; B = Outer loop counter

OUTER: LXI H, 4201    ; Point HL to first number

MOV D, C              ; D = Inner loop counter

INNER: MOV A, M       ; A = M[i]

INX H

CMP M                ; Compare A with M[i+1]

JNC SKIP             ; If A >= M[i+1], skip swap

MOV E, M             ; Else, swap

MOV M, A

DCX H

MOV M, E

INX H

SKIP: DCR D

JNZ INNER

DCR B

JNZ OUTER

HLT

### Sample Input (Memory Starting at 4200H):

| Address | Data |
| --- | --- |
| 4200H | 05H |
| 4201H | 3AH |
| 4202H | 12H |
| 4203H | 41H |
| 4204H | 1CH |
| 4205H | 2FH |

### Expected Output (Descending):

| Address | Data |
| --- | --- |
| 4201H | 41H |
| 4202H | 3AH |
| 4203H | 2FH |
| 4204H | 1CH |
| 4205H | 12H |

Result:  The 8085 assembly language program was successfully executed to sort a given set of numbers in descending order.